Perceptron Branch Predictor for PicoRV32-based SoC

Arthur Kim, Derek Kropp, Chris Rumsey

Department of Electrical and Computer Engineering, University of Arizona, Tucson, AZ

Abstract

This project implements a perceptron inspired branch predictor, which is wired in as an add-module to a RISC-V PicoRV32 cpu.  This combined chip also contains 2 KB of SRAM added via the OpenRAM configuration.  This stack is then implemented from RTL to GDSII with the open source SiliconCompiler program.

Keywords (optional): RISC-V, PicoRV32, System-on-Chip, SiliconCompiler, Skywater PDK, custom hardware module, ASIC, Branch Prediction, Perceptron.

Introduction

This demonstrates the comprehensive workflow of designing, synthesizing, and integrating a basic System-on-Chip (SoC) using the PicoRV32 RISC-V CPU core with an added Perceptron style branch prediction module

RISC-V processors are increasingly popular due to their open-source architecture, facilitating customization and wide adoption in academic and industrial settings. PicoRV32 represents a compact and efficient RISC-V implementation optimized for resource-constrained environments. This framework is extensible, allowing demonstration of capabilities by adding this custom branch predictor.

Project Design & Architecture

The branch predictor consists of 2 parts.  A simple 3 bit global branch history results, containing the binary result of the last 3 branches.  It also contains a hashed perception prediction table.  This table contains 512 32 bit entries.  The program counter address when it reaches a branch is hashed with an XOR of two 9 bit values within the 32 bits.  This reduces the 32 bit address to a 9 bit access to the branch perceptron table.  The 32 bits of the table contain 4 8 bit components, 3 weights and 1 bias.  When the table entry is accessed, the result is calculated by summing a score where score = bias + weight0 * global history bit0 + weight1 * global history bit1+weight2* global history bit2.

If the score is positive it predicts branch taken.  If negative, it predicts not taken.  The result of the recent branch is then fed back to update the weights of the perceptron.

Methodology

The Paper employed SiliconCompiler, a modern, Python-based framework for automated ASIC design. This tool facilitates rapid development by managing synthesis, placement, routing, and verification in a unified workflow.

Phase 1: Building the PicoRV32 Core

Initially, the PicoRV32 Verilog code was downloaded from its official GitHub repository. A build script (picorv32.py) was created in Python utilizing SiliconCompiler to orchestrate the RTL-to-GDS-II design flow, as demonstrated in Fig. 1. SiliconCompiler was executed remotely using the Skywater130 standard cell library to synthesize the core, ensuring functional correctness by verifying the Monte Carlo analyses shown in Table I.

Phase 2: Adding the Perceptron Branch Predictor

To demonstrate the extensibility of the PicoRV32 CPU to a SOC, a peripheral branch predictor was added.  This was implemented in Verilog (neural_bht.v), and wired into the top-level module (picorv32_bp_top.v).  The top level was then modified to include the new file, and modified to add a size upgrade to accommodate the new module, as well as defining the tool chain for local OpenROAD implementation.

Implementation

The PicoRV32 CPU core was synthesized using SiliconCompiler's RTL-to-GDSII flow with the open-source Skywater130 PDK.   This tool chain was run locally using the OpenROAD tool kit, validated with the similar SiliconCompiler output.

Results

[The complete synthesis and layout process took approximately 50 minutes to execute. The core-only PicoRV32 design (Fig. 3) achieved an area of 803,942 µm² and a maximum operational frequency (Fmax) of 37.369 MHz. Upon integrating the 2 KB SRAM macro, the total area increased to 956,755 µm², and the maximum frequency slightly decreased to 35.956 MHz (Fig. 4). The synthesis and place-and-route procedures completed successfully, with no critical errors or warnings encountered. Detailed utilization metrics, timing analyses, and further insights into the process can be reviewed through the SiliconCompiler dashboard and detailed log outputs as depicted in (Fig. 2).]

Analysis

[The successful integration of the SRAM macro into the PicoRV32-based SoC validated the efficiency and practicality of the SiliconCompiler framework. A minor performance trade-off, with maximum frequency decreasing from 37.369 MHz to 35.956 MHz, was primarily due to increased routing complexity from macro placement but remained within acceptable limits. Strategic macro placement effectively minimized routing congestion, facilitating error-free synthesis and layout completion. The efficient 50-minute end-to-end workflow utilizing the open-source Skywater130 PDK and OpenRAM configurations highlights the capability of open-source tools for rapid ASIC prototyping, offering a reliable foundation for subsequent enhancements.]

Conclusion and Future Work

[This paper effectively illustrates the viability and advantages of using SiliconCompiler alongside open-source tools and IP for efficient ASIC design. The generated SoC provides a robust platform, facilitating future integration of additional peripherals and custom hardware accelerators.]

References

[1] YosysHQ, "PicoRV32: A Size-Optimized RISC-V CPU Core", GitHub Repository, https://github.com/YosysHQ/picorv32

[2] SiliconCompiler Documentation, https://docs.siliconcompiler.com

[3] ECE 407/507 Project Description, University of Arizona, 2025.

[4] RISC-V Foundation, "The RISC-V Instruction Set Manual", https://riscv.org.

| Fig. 1. Run screen of the RTL-to-GDS-II design flow test of the Picorv32 Verilog file. | TABLE I. Monte Carlo Analyses each stage in the RTL-to-GDS-II flow. Cell area (um^2); utilization (%); peak power (mw); setup slack (ns); fmax (MHz); total time (s). syn0; 336.573; -; -; -; -; 04.182. floorplan. init0; 336.573; 22.342; 0.037; 5.259; -; 07.715. floorplan.pin_ placement0; 356.592; 23.671; 0.037; 5.156; -; 19.532. place. global0; 370.355; 24.585; 0.037; 5.114; 204.675; 25.529. place. detailed0; 370.355; 24.585; 0.037; 5.1; 204.089; 36.235. cts.clock_tree_ synthesis0; 389.123; 25.831; 0.057; 5.072; 202.935; 01:02.1. cts.repair_ timing0; 385.37; 25.581; 0.057; 4.827; 193.314; 01:09.1. route.global0; 385.37; 25.581; 0.065; 4.549; 183.467; 01:19.4. route.detailed0; 385.37; 25.581; 0.055; 4.946; 197.88; 01:41.1. write.views0; 385.37; 25.581; 0.056; 4.855; 194.356; 01:55.6. |
| --- | --- |
| TABLE II. Final Floorplan Area, Power, and utilization metrics. Metrics; floorplan.tapcell0. errors; 0. warnings; 2. Total area (um^2); 2500. Stdcell area (um^2); 356.592. utilization (%); 23.671. peak power (mw); 0.056. cells; 41. I/O pins; 3. nets; 30. Task time (s); 3.557. Total time (s); 11.997. | Fig. 2. Detailed monitoring for peak power throughout the workflow via the SiliconCompiler dashboard. |
| Fig. 3. GDSII extracted Layout with Picrorv32 details. | Fig. 4. GDSII extracted Layout with branch predictor integrated to Picorv32 |

a